Гусев Виталий Евгеньевич

2.3.5

Description of the Scientific Work

Topic: Modeling of a True Random Number Generator Based on Ternary Logic Elements

Introduction and Relevance

The security of modern cryptographic systems depends critically on high‑quality randomness. True Random Number Generators (TRNGs) that exploit physical entropy sources—such as jitter in digital circuits—offer unpredictability essential for cryptographic keys, nonces, and authentication protocols. While most existing TRNGs are based on binary logic, ternary (three‑valued) logic presents potential advantages in circuit density, power efficiency, and compatibility with emerging nanotechnologies. My research focuses on developing a theoretical model for a novel TRNG constructed from identical ternary logic gates connected in a feedback configuration.

Aims and Objectives

The main goal is to mathematically model and analyze the behavior of a jitter‑based TRNG built from ternary combinational gates. Specific objectives are:

- To formulate a continuous‑time Markov chain model assuming independent exponentially distributed gate delays.

- To derive the Erlang‑type differential equations governing the generator’s state probabilities.

- To find the stationary distribution and convergence rate for different numbers of gates and circuit topologies.

- To evaluate the uniformity and statistical properties of the output random numbers.

Methodology

The generator is modeled as a Markov process where each state is a vector of ternary output values. Transition rates are determined by the exponential delay assumption and a ternary logic function (e.g., the function satisfying that the output always differs from both inputs). The master equation is solved analytically for small N and numerically for larger N using matrix exponentiation. Simulation in Python is used to validate theoretical results and to study convergence speed.

Expected Results and Contribution

Key anticipated outcomes include:

- Proof that for a ring of N≥3 gates, the stationary distribution is non‑uniform, yet each individual gate’s output is uniformly distributed over {-1,0,1}.

- Identification of unreachable states and practical ways to circumvent them.

- Quantitative estimation of the clock period needed to achieve near‑uniform output.

The work extends the theory of jitter‑based TRNGs from binary to ternary logic, providing a foundation for designing compact hardware random number generators. This contributes to the development of physically secure random sources for post‑quantum cryptographic systems and other security‑sensitive applications.